PROJECT OVERVIEW

HQ-Map is a novel hierarchical TD(0) Q-learning framework that dynamically maps 32 tasks onto an 8×8 NoC SoC across three automotive driving contexts — City (steps 1–300), Highway (301–700), and Parked (701–1000). Unlike prior work (Q-Thermal, TTQR, FLEA), HQ-Map jointly optimises communication latency, thermal safety, and ISO 26262 ASIL-D safety-critical task proximity in a single reward function. The framework uses spectral graph-cut clustering with critical-task co-location bias, a corrected Bellman update with γ = 0.9, and introduces SCSR — a novel safety constraint satisfaction metric not present in any prior NoC scheduling work.

| 8×8 NoC Mesh · 64 Cores | 32 Tasks per Step | 30,000 Simulation Steps | 20 RL States (5 load × 4 thermal) |
| --- | --- | --- | --- |

TECHNICAL APPROACH

| Two-Level Hierarchical Q-Learning Level 1 — Coarse: Q[20×4 zones]. Agent selects NoC quadrant for each task cluster using joint thermal+load state index (1..20). Level 2 — Fine: Q[20×16 cores]. Agent selects specific core within chosen zone using zone occupancy level. Bellman TD(0) update: Q(s,a) ← Q(s,a) + α·[r + γ·max Q(s′) − Q(s,a)] where α=0.1, γ=0.9 Spectral clustering: Normalised Laplacian eigenvectors. Critical-task edges boosted 5× to enforce co-location in same zone. | Reward Function & Metrics Reward = −0.5·hop_norm − 1.0·thermal_pen − 2.0·safety_pen + 2.0 bonus Thermal penalty: 0 below 75°C ¦ linear 75–85°C ¦ quadratic above 85°C Safety penalty: Fraction of critical pairs >3 hops apart (normalised by pair count) SCSR (novel): Safety Constraint Satisfaction Rate = satisfied_pairs / total_critical_pairs. First ISO 26262 quantification for NoC task scheduling. |
| --- | --- |

COMPARATIVE RESULTS — TABLE I (30,000 SIMULATION STEPS)

| Method | Avg Reward | SCSR (%) | Max Temp | Latency | Violations |
| --- | --- | --- | --- | --- | --- |
| HQ-Map (Ours) | +1.291 | 70.4% | 69.0°C | 1476 | 0 |
| Flat Q (FLEA) | +0.474 | 33.3% | 69.0°C | 3863 | 0 |
| Q-Thermal style | +0.304 | 24.7% | 69.0°C | 3885 | 0 |
| Greedy NN | +0.751 | 44.0% | 61.3°C | 2653 | 0 |
| Round-Robin | +0.363 | 27.6% | 68.7°C | 3923 | 0 |

Key results: HQ-Map leads on Reward (+1.291), SCSR (70.4%), and Latency (1476). Highway learning delta +1.896 — 3.4× better than Flat Q. SCSR is 2.8× better than Q-Thermal. Zero thermal violations, max 69.0°C.

| NOVELTY vs PRIOR ART. Gap; HQ-Map Solution. Mode-aware RL; City/Highway/Parked transitions in Q-table update — absent in all prior work.. Safety reward; w_safety=2.0 co-optimised with latency and thermal — not in Q-Thermal or TTQR.. Two-level hierarchy; Zone→core decomposition keeps Q-tables tractable for 64 cores.. SCSR metric; First ISO 26262 compliance metric for NoC scheduling.. | SDG ALIGNMENT. SDG; Project Contribution. SDG 3 — Good Health; SCSR 70.4% (vs 24.7%) satisfies ISO 26262 ASIL-D; reduces road fatalities.. SDG 9 — Industry; 62% latency reduction over Round-Robin enables efficient automotive SoC infrastructure.. SDG 11 — Sustainable Cities; Mode-aware ADAS scheduling for safe smart-city autonomous vehicle deployment.. SDG 13 — Climate; Zero thermal violations (max 69°C) minimises cooling energy, lowering CO₂ emissions.. |
| --- | --- |