HQ-Map: Hierarchical Q-Learning for Task Mapping

on a Thermal-Aware Smart Vehicle SoC

Project Summary  |  Implemented in GNU Octave

# Project Overview

HQ-Map is a hierarchical reinforcement learning framework for real-time task mapping on a Network-on-Chip (NoC) based System-on-Chip (SoC) designed for automotive/smart vehicle applications. The system dynamically maps 32 concurrent tasks onto a 64-core (8×8) NoC mesh, optimising simultaneously for communication latency, thermal safety, and load balance — adapting its strategy as the vehicle's operational mode changes.

# Problem Statement

Modern automotive SoCs must execute diverse, safety-critical workloads (perception, ADAS, infotainment) under strict thermal and latency constraints. Static task mapping cannot adapt to dynamic workloads, while naive RL approaches scale poorly to large multi-core systems. The core challenges are:

- State-space explosion: a 64-core system with 32 tasks has an intractable flat mapping space

- Conflicting objectives: minimising hop-count latency, preventing thermal hotspots, and maintaining load balance must be balanced simultaneously

- Safety-criticality: ADAS-type tasks have hard communication proximity constraints (≤3 hops) that cannot be violated

- Dynamic workloads: City driving, highway cruising, and parked modes impose very different task mixes and traffic patterns

# Proposed Approach: HQ-Map

HQ-Map decomposes the mapping problem into a two-level hierarchy:

- Coarse Level (Zone Selection): A coarse Q-table (20 states × 4 zones) selects which NoC quadrant to place each task cluster into, using a 20-state encoder combining thermal load and spatial occupancy.

- Fine Level (Core Selection): A fine Q-table (20 states × 16 cores/zone) selects the exact core within the chosen zone, guided by local zone occupancy.

- Spectral Task Clustering: Tasks are pre-grouped using normalised Laplacian spectral clustering on the communication graph, with a 5× co-location bias for safety-critical (Type-1) task pairs.

- Multi-Objective Reward: Reward = −0.5·hop_cost − 1.0·thermal_penalty − 2.0·safety_penalty + 2.0 (completion bonus). Priority ordering: Safety > Thermal > Latency, reflecting automotive standards.

- Correct TD(0) Q-Updates: Bellman bootstrapping Q(s,a) ← Q(s,a) + α[r + γ·max Q(s',a') − Q(s,a)] is applied at both hierarchy levels with ε-greedy exploration (ε: 0.5 → 0.05).

# Comparison with Prior Art

HQ-Map is benchmarked against four baselines over 1,000 simulation steps across three driving modes (City: steps 1–300, Highway: 301–700, Parked: 701–1000):

| Method | Basis | Key Limitation |
| --- | --- | --- |
| HQ-Map (Ours) | Hierarchical TD Q-learning + Spectral Clustering | — |
| Flat Q (FLEA-style) | Single flat Q-table, same reward/state | No zone/core hierarchy; slow to converge |
| Q-Thermal style | Q-learning on thermal state only | Ignores load balance and safety constraints |
| Greedy Nearest-Neighbour | Place each task on closest free core to communicating partners | No thermal or load awareness |
| Round-Robin | Sequential placement across all cores | Lower-bound baseline; no optimisation |

A novel Safety Constraint Satisfaction Rate (SCSR) metric — fraction of safety-critical task pairs placed within 3 hops — is introduced as a per-mode evaluation measure not present in prior work.

# Key Metrics & Expected Outcomes

- Peak core temperature: lower is better; critical threshold 85 °C

- Communication hop cost: weighted Manhattan hop sum across all communicating task pairs

- Load balance: standard deviation of task count across the 4 NoC zones (lower = more balanced)

- SCSR: fraction of safety-critical communicating pairs satisfying the ≤3-hop proximity constraint

- Cumulative reward: overall policy quality across all three driving modes

# Technical Implementation

| Parameter | Value |
| --- | --- |
| Platform | GNU Octave (MATLAB-compatible, no toolboxes required) |
| NoC topology | 8×8 mesh, 64 cores, 4 clusters (quadrants) |
| Task load | 32 tasks, 3 types: Safety-critical (Type 1), Compute (Type 2), Background (Type 3) |
| Heat model | Per-step heat injection [6.0, 3.0, 1.5] °C by type; 25% dissipation; ambient 45 °C |
| RL parameters | α=0.1, γ=0.9, ε: 0.5→0.05 (decay 0.9935 per step) |
| State encoding | 20-state coarse (thermal + occupancy), 20-state fine (zone fill) |
| Simulation length | 1,000 steps; mode reclustering at steps 1, 301, 701 |

# Key Contributions

- First hierarchical Q-learning formulation for NoC task mapping that jointly optimises latency, thermal safety, and load balance

- Spectral clustering with safety-task co-location bias — groups critical tasks to reduce SCSR violations before RL even begins

- Novel SCSR metric for per-mode safety constraint evaluation in automotive SoC mapping

- Full Octave-compatible implementation with no external toolboxes — reproducible on any open-source scientific computing platform

- Comparative evaluation against four prior-art methods across three realistic automotive operating modes

HQ-Map Project Summary  —  Implemented in GNU Octave  —  hq_map_phase2_main.m / run_comparison.m